DEPARTMENT OF ARTIFICIAL INTELLIGENCE AND DATA SCIENCE

INNOVATION IN TEACHING REPORT

Implementation of Real-Time Clock (RTC) Using Verilog HDL on Spartan-3 FPGA (XC3S400-PQ208) for Digital Design Laboratory

By

Mr. Durairaji V

Assistant Professor

Department of Artificial Intelligence and Data Science

St. Joseph’s College of Engineering

Chennai - 600119

https://github.com/durairaji1984/Implementation-of-Real-Time-Clock-RTC-Using-Verilog-HDL/tree/main

- Innovation Type

Hardware-Based Experiential Learning using FPGA Prototyping

The innovation involves demonstrating a complete Real-Time Clock (RTC) system implemented using Verilog HDL on a Xilinx Spartan-3 FPGA (XC3S400-PQ208) development board. Students learn digital design concepts through real-time hardware execution rather than simulation alone.

- Course Details

| Particular | Details |
| --- | --- |
| Course Name | Digital Principles and Logic Design |
| Laboratory | Digital Design Laboratory |
| Topic | Real-Time Clock using FPGA |
| Platform | Xilinx Spartan-3 FPGA XC3S400-PQ208 |
| Programming Language | Verilog HDL |
| Software Tool | Xilinx ISE Design Suite |
| Clock Frequency | 4 MHz |
| Display Device | 6 - Digit Seven Segment Display |

- Objectives

The innovative teaching activity aims to:

- Demonstrate real-time digital system design.

- Introduce FPGA-based hardware implementation.

- Explain clock division and timing circuits.

- Illustrate counter design using Verilog HDL.

- Familiarize students with synthesis, simulation, and hardware verification.

- Bridge the gap between theoretical digital electronics and industrial FPGA applications.

- Need for Innovation

Traditional teaching methods explain counters, registers, and timing circuits theoretically. Students often find it difficult to visualize how these concepts operate in real-time.

The FPGA-based RTC model provides:

Hands-on learning experience.

- Real-time observation of clock operation.

- Understanding of hierarchical digital design.

- Exposure to industrial FPGA technology.

- Improved problem-solving and debugging skills.

- Description of the Innovative Model

The RTC system was designed using Verilog HDL and implemented on the Spartan-3 FPGA board.

The system:

- Accepts a 4 MHz clock input.

- Generates a 1-second pulse using a clock divider.

- Maintains:

  - Seconds Counter (00–59)

  - Minutes Counter (00–59)

  - Hours Counter (00–23)

- Displays time on a six-digit seven-segment display.

- Resets automatically after 23:59:59.

The implementation uses:

- Cascaded BCD Counters

- Clock Divider

- Seven Segment Decoder

- Display Multiplexing Technique

The Verilog code creates a 1-second enable pulse from the 4 MHz clock and updates the counters sequentially.

PROGRAM:

module finalclock_verilog(sgout,dis,load,reset,clock);

output [7:0]sgout;

output [5:0]dis;

input reset,clock,load;

reg [3:0]sec1_rg,sec2_rg,min1_rg,min2_rg,hr1_rg,hr2_rg;

reg [21:0] pulsegen ;

reg [2:0]cnk2;

reg [3:0]mout;

reg [7:0]sgout_rg;

reg [5:0]dis_sig;

wire [3:0]sec1,sec2,min1,min2,hr1,hr2;

wire  tc1,tc2,tc3,tc4,tc5,tc6,enable;

//reg tc,tc1,tc2,tc3,tc4,tc5,tc6,enable;

//reg sec1_rg(3:0),sec2_rg(3:0),min1_rg(3:0),min2_rg(3:0),hr1_rg(3:0);

//reg hr2_rg(3:0),pulsegen(21:0),sel(2:0);

//reg mout(3:0),sgout(7:0),ck1(22:0),cnk2(2:0);

//*************************** Pulse Generator ******************

always@(posedge clock or posedge reset)

begin

if (reset)

pulsegen <= 22'b0;

else

begin

//if (pulsegen == 22'b1111010000100100000000)

if (pulsegen == 22'b111101)

pulsegen <= 22'b0;

else

pulsegen <= pulsegen + 1;

end

end

//Enable signal to generate 1-sec pulse for sec1 counter

//assign enable = (pulsegen == 22'b1111010000100100000000) ; //enable signal for sec1 counter

assign enable = (pulsegen == 22'b111101) ; //enable signal for sec1 counter

//************************ Second_cntr1 *************************

always@(posedge clock or posedge reset)

begin

if (reset )

sec1_rg <= 4'b0000;

else if (load) begin

sec1_rg <= 4'b0100;

end

else

begin

if (enable)

begin

if (sec1_rg == 4'b1001)

sec1_rg <= 4'b0000;

else

sec1_rg <= sec1_rg + 1;

end

end

end

assign sec1 = sec1_rg;

//------------------tc1 signal to start sec2 counter---------------------------

assign tc1 = (sec1_rg == 4'b1001) && (enable == 1);

//--************************* Second_cntr2 ***********************

always@(posedge clock or posedge reset)

begin

if (reset )

sec2_rg <= 4'b0000;

else if (load) begin

sec2_rg <= 4'b0100;

end

else

begin

if (tc1)

begin

if (sec2_rg == 4'b0101) begin

sec2_rg <= 4'b0000;

end

else begin

sec2_rg <= sec2_rg + 1;

end

end

end

end

assign sec2 = sec2_rg;

//-------------------------tc2 signal to start min1 counter-------------------

assign tc2 = (sec2_rg == 4'b0101) && (tc1 == 1);

//--************************ Minute_cntr1 *************************

always@(posedge clock or posedge reset)

begin

if (reset )

min1_rg <= 4'b0000;

else if (load) begin

min1_rg <= 4'b0100;

end

else

begin

if (tc2)

begin

if (min1_rg == 4'b1001) begin

min1_rg <= 4'b0000;

end

else begin

min1_rg <= min1_rg + 1;

end

end

end

end

assign min1 = min1_rg;

//------------------------tc3 signal to start min2 counter--------------------

assign tc3 = (min1_rg == 4'b1001) && (tc2 == 1);

//--************************ Hour_cntr1 *************************

always@(posedge clock or posedge reset)

begin

if (reset)

min2_rg <= 4'b0000;

else if (load) begin

min2_rg <= 4'b0100;

end

else

begin

if (tc3)

begin

if (min2_rg == 4'b0101) begin

min2_rg <= 4'b0000;

end

else

begin

min2_rg <= min2_rg + 1;

end

end

end

end

assign min2 = min2_rg;

//---------------------tc4 signal to start hr1 counter--------------------------

assign tc4 = (min2_rg == 4'b0101) && (tc3 == 1);

//--************************ Hour_cntr1 *************************

always@(posedge clock or posedge reset)

begin

if (reset)

hr1_rg <= 4'b0000;

else if (load) begin

hr1_rg <= 4'b0001;

end

else

begin

if (tc6)

begin

hr1_rg <= 4'b0000;

end

else

begin

if (tc4)

begin

if (hr1_rg == 4'b1001) begin

hr1_rg <= 4'b0000;

end

else

begin

hr1_rg <= hr1_rg + 1;

end

end

end

end

end

assign hr1 = hr1_rg;

//---------------------tc5 signal to start hr2 counter--------------------------

assign tc5 = (hr1_rg == 4'b1001) && (tc4 == 1);

//------------------------tc6 signal to reset at 23:59:59--------------------------

assign tc6 = (hr2_rg == 4'b0010) && (hr1_rg == 4'b0011) && (tc4 == 1);

//--************************ Hour_cntr2 *************************

always@(posedge clock or posedge reset)

begin

if (reset)

hr2_rg <= 4'b0000;

else if (load) begin

hr2_rg <= 4'b0000;

end

else

begin

if (tc6)

begin

hr2_rg <= 4'b0000;

end

else

begin

if (tc5)

begin

if (hr2_rg == 4'b0010) begin

hr2_rg <= 4'b0000;

end

else

begin

hr2_rg <= hr2_rg + 1;

end

end

end

end

end

assign hr2 = hr2_rg;

always@(posedge pulsegen[9] or posedge reset)

begin

if (reset)

cnk2 <= 3'b0;

else

begin

if (cnk2 == 3'b101)

cnk2 <= 3'b0 ;

else

cnk2 <= cnk2 + 1;

end

end

always@(cnk2)

case (cnk2)

3'b000 : mout <= sec1;

3'b001 : mout <= sec2;

3'b010 : mout <= min1;

3'b011 : mout <= min2;

3'b100 : mout <= hr1;

3'b101 : mout <= hr2;

endcase

always@(mout)

case (mout)

4'b0000 : sgout_rg <= 8'b11000000;

4'b0001 : sgout_rg <= 8'b11111001;

4'b0010 : sgout_rg <= 8'b10100100;

4'b0011 : sgout_rg <= 8'b10110000;

4'b0100 : sgout_rg <= 8'b10011001;

4'b0101 : sgout_rg <= 8'b10010010;

4'b0110 : sgout_rg <= 8'b10000010;

4'b0111 : sgout_rg <= 8'b11111000;

4'b1000 : sgout_rg <= 8'b10000000;

4'b1001 : sgout_rg <= 8'b10011000;

endcase

always@(cnk2)

case (cnk2)

3'b000 : dis_sig <= 6'b111110;

3'b001 : dis_sig <= 6'b111101;

3'b010 : dis_sig <= 6'b111011;

3'b011 : dis_sig <= 6'b110111;

3'b100 : dis_sig <= 6'b101111;

3'b101 : dis_sig <= 6'b011111;

endcase

assign sgout =  ~ sgout_rg;

assign dis = ~ dis_sig;

endmodule

- Hardware Setup

- Spartan-3 FPGA Board (XC3S400-PQ208)

- 4 MHz Crystal Oscillator

- Six-Digit Seven Segment Display Board

- Ribbon Cable Interface

- Power Supply

- FPGA Pin Assignments

Important pin mappings from the UCF (User Constraints File) file:

net clock loc = p181;

net reset loc = p182;

net load loc = p33;

net sgout<7> loc =p199;

net sgout<6> loc =p198;

net sgout<5> loc =p197;

net sgout<4> loc =p196;

net sgout<3> loc =p194;

net sgout<2> loc =p191;

net sgout<1> loc=p190;

net sgout<0> loc=p189;

net dis<5> loc = p10;

net dis<4> loc = p11;

net dis<3> loc = p12;

net dis<2> loc = p13;

net dis<1> loc = p15;

net dis<0> loc = p16;

- Working Principle

Step 1: Clock Division

The FPGA receives a 4 MHz clock.

A pulse generator divides the clock and generates a 1-second enable signal.

Step 2: Seconds Counter

- Counts from 00 to 59.

- Generates carry signal tc1.

Step 3: Minutes Counter

- Increments whenever seconds reach 59.

- Counts from 00 to 59.

Step 4: Hours Counter

- Counts from 00 to 23.

- Resets automatically after 23:59:59.

Step 5: Display Multiplexing

The six seven-segment displays are activated one after another at high speed to create continuous visualization.

- RTL (Register-Transfer Level) Schematic

The RTL design contains:

- Clock Divider Module

- Second Counter Module

- Minute Counter Module

- Hour Counter Module

- Seven Segment Decoder

- Display Multiplexer

The synthesized RTL schematic clearly shows the hierarchical implementation of the RTC design.

- Simulation Results

Observed outputs:

Time Display

01:47:03

- Hardware Implementation

- Learning Outcomes

After performing this activity, students are able to:

- Design synchronous sequential circuits.

- Implement clock dividers.

- Develop Verilog HDL modules.

- Perform FPGA synthesis and implementation.

- Analyze RTL schematics.

- Verify functionality using simulation tools.

- Interface FPGA with seven-segment displays.

- Develop real-time embedded digital systems.

- Outcome-Based Education (OBE) Mapping

| POs | Contribution |
| --- | --- |
| PO1 | Engineering Knowledge |
| PO2 | Problem Analysis |
| PO3 | Design of Solutions |
| PO4 | Investigation |
| PO5 | Modern Tool Usage |
| PO12 | Lifelong Learning |

- Conclusion

The Real-Time Clock (RTC) implementation using Verilog HDL on Spartan-3 FPGA XC3S400-PQ208 is an effective innovative teaching model that transforms theoretical concepts of counters, timing circuits, and sequential logic into a practical real-time hardware application. The activity enhances student engagement, improves FPGA design skills, and provides industrial-level exposure to digital system development.

Innovation Outcome: Students achieved a better understanding of digital design concepts through real-time FPGA implementation, simulation verification, RTL analysis, and hardware testing.